3. Product Design Specification (PDS)

The design and manufacturing boundaries of the PlanetSort: OrbitQuest mechatronic system are strictly governed by the following engineering criteria, established in accordance with course constraints and childhood safety standards:

| Parameter ID | Category | Performance Requirement | Quantitative Target Metric | Priority (D/W) | Engineering Justification & Source |
| --- | --- | --- | --- | --- | --- |
| PDS-01 | Performance | Primary Functional Cycle | Complete autonomous color-based scanning, helical elevation, and parabolic projectile tracking. | D | Fulfills core mechatronic module requirements defined by the teaching committee. |
| PDS-02 | Performance | Closed-Loop Path Execution | 1 single ball loading point; 100 percent internal containment tracking throughout the complete operational loop. | D | Eradicates loose rolling pieces and ensures total containment of loose parts during the full loop. |
| PDS-03 | Mechanical | Ball Classification (Sorting) | Automated color identification via wavelength checks to separate balls into 2 divergent channels. | D | Driven by a TCS3200 sensor array acting on dual SG90 micro-servo flipper gates. |
| PDS-04 | Mechanical | Vertical Lifting Mechanism | Enclosed helical vertical transit lifting columns safely transporting marbles. | D | Employs an Archimedes spiral screw coupled to a NEMA 17 stepper motor running via an A4988 driver. |
| PDS-05 | Mechanical | Projectile Launch Subsystem | Friction-driven twin counter-rotating projection engine angled at a fixed incline. | D | Calibrated via PWM duty cycles to achieve an exit velocity of v0 = 4.15 m/s at a fixed 30-degree launch angle. |
| PDS-06 | Control | Embedded Processing Nodes | Dual-microcontroller processing grid mapping tasks across independent domains. | D | Deploys 2 x ESP32 DevKitC boards communicating wirelessly via low-latency peer-to-peer ESP-NOW protocol. |
| PDS-07 | Control | Local User Interface | Direct visual character menu terminal output (Seven-Segment configurations are strictly prohibited). | D | Employs 2 x onboard 16x4 character I2C LCD displays showing real-time subsystem state tracking. |
| PDS-08 | Control | Remote Monitoring Terminal | Wireless graphical monitoring software providing score and color parameter logging. | W | Achieved through a custom Processing.io application interacting via integrated Bluetooth Serial Port Profile. |
| PDS-09 | Electrical | Power Infrastructure | Single, unified power input terminal block layout supplying the entire backplane. | D | Eradicates independent secondary battery networks; safely driven by a single 12V, 5A DC adapter power source. |
| PDS-10 | Electrical | Onboard Voltage Regulation | Local voltage step-down buses to distribute isolated logic and actuator rails. | D | Integrates LM2596-based buck converter circuits on the custom PCB to step down 12V input to a stabilized 5V rail. |
| PDS-11 | Electrical | Custom Manufacturing | Clean signal routing and integration of system hardware components. | D | Mandates a minimum of 1 custom-manufactured PCB serving as the power distribution and circuit protection hub. |
| PDS-12 | Physical | Volumetric Footprint Bounds | Total external packaging bounds must remain within a compact structural envelope. | D | Maximum allowable boundaries of 100 cm x 100 cm x 100 cm; nominal layout optimized at 70 cm x 60 cm. |
| PDS-13 | Physical | Structural Composition | Non-toxic, rigid, drop-resistant material profiles with post-processed sanded facings. | D | Utilizes child-safe, deburred 3D-printed PLA mechanical paths mounted securely on a smooth 6 mm MDF baseboard. |
| PDS-14 | Physical | Structural Modularity | Subsystem mechanical arrays must cleanly decouple for maintenance. | D | Divided into 3 distinct modules (Sorting, Lifting, Shooting) using dedicated, polarized JST cable harnesses. |
| PDS-15 | Safety | Hazard & Shock Proofing | 100 percent concealment of electronic terminals and moving columns from user contact. | D | All electrical wiring is routed through integrated internal tracks; the helical lifter is encased inside clear acrylic. |
| PDS-16 | Safety | Ingestion Hazard Control | Planetary marble diameter sizing must remain strictly above choke testing cylinder limits. | D | Marble diameters are constrained between 35 mm and 40 mm to achieve compliance with global small parts toy safety criteria. |
| PDS-17 | Safety | Overcurrent Circuit Defense | Automatic hardware-level current trip points to protect logic lines if a mechanism stalls. | D | Features an inline, resettable polymeric 5A PTC fuse positioned immediately after the primary 12V DC power jack. |
| PDS-18 | Safety | Emergency System Shutdown | Instantaneous operational override path to halt all active actuators concurrently. | D | Physical, high-priority tactile push buttons configured to force immediate low-level hardware firmware interrupts. |

The D/W Priority Filter: By explicitly labeling safety and course specifications as absolute a Demand (D) and a Wish (W), you show the panel that risk mitigation was treated as a primary engineering objective from day one.